Q40 Fencing & Gates Specification

Description:			Perimeter Security Fencing Panel System

Standards:			BS1722-14:2007

BS 4102:1998

BS EN 10244-2:2001

BS EN 10210-2:1997

BS EN 10346:2009

BS EN 13438:2005

Manufacturer:		Alexandra Security Ltd

estimating@alexandrasecurity.com

www.alexandrasecurity.com

01892 833 001

Product:			Protek® 656 Clip

Product Type:			Twin-Wire Boundary Fencing System

Panel Type:	Mesh & Wire. Galvanised and powder coated to minimum 100 micron.

Panel Width:	2,506mm

Wire Diameter:	5mm vertical, 6mm horizontal

Mesh Size:	50 x 200mm mesh

Available Heights:	1.2m to 3.0m

Finish:	Any RAL colour. Marine coating finish available.

Panel Installation:	Max 50mm above ground level. Available with dig-in or base-plated post options.

Topping Options:	Cranked or straight extensions for mesh, barbed or razor wire

Posts: 	Supplied with plastic cap.

Post Dimensions:	60 x 60mm

Post Centres:	2,520mm

Fixings:	Metal clips. M8 x 40 Fourtress tamper resistant security screw.

Warranty:	15-year guarantee on performance and appearance.

Notes:	Information correct at date of publication. Alexandra Security Ltd reserves the right to change specifications at any time. E&OE.