Q40 Fencing & Gates Specification

Description:			Perimeter Security Fencing Panel System

Standards:			LPS 1175 SR2 (B3)

BS1722-14:2007

BS 4102:1998

BS EN 10244-2:2001

BS EN 10210-2:1997

BS EN 10346:2009

BS EN 13438:2005

Secured By Design – Police Preferred Specification

Manufacturer:		Alexandra Security Ltd

estimating@alexandrasecurity.com

www.alexandrasecurity.com

01892 833 001

Product:			Protek® Elite Defender

Product Type:			SR-2 Rated Security Fencing System

Panel Type:	Mesh & Wire. Galvanised and powder coated to minimum 100 micron.

Panel Width:	2,525mm

Wire Diameter:	10mm vertical, 4mm horizontal

Mesh Size:	12.7 x 76.2mm mesh

Available Heights:	2.4m to 5.2m

Finish:	Any RAL colour. Marine coating finish available.

Panel Installation:	Max 50mm above ground level. Available with dig-in or base-plated post options.

Topping Options:	Cranked or straight extensions for mesh, barbed or razor wire

Posts: 	Supplied with plastic cap.

Post Dimensions:	80 x 80mm minimum depending on site conditions

Post Centres:	2,535mm

Fixings:	80 x 3mm vertical clamp bar. 40 x 5mm horizontal clamp bar, M8 cup square security bolt. Security nuts and washers.

Warranty:	15-year guarantee on performance and appearance.

Notes:	Information correct at date of publication. Alexandra Security Ltd reserves the right to change specifications at any time. E&OE.