Q40 Fencing & Gates Specification

Description:			Perimeter Security Fencing Panel System

Standards:			BS1722-14:2007

BS 4102:1998

BS EN 10244-2:2001

BS EN 10210-2:1997

BS EN 10346:2009

BS EN 13438:2005

Manufacturer:		Alexandra Security Ltd

estimating@alexandrasecurity.com

www.alexandrasecurity.com

01892 833 001

Product:			SolarGard Pulsar Plus

Product Type:			V-Mesh & Timber Boundary Fencing System

Panel Type:	Mesh & Wire. Galvanised and powder coated to minimum 100 micron.

Panel Width:	2,510mm

Wire Diameter:	5mm

Mesh Size:	55 x 200mm mesh

Timber:	19 x 50mm timber slats with planned finish. Pressure treated finish. Height matches mesh panel.

Number of ‘V’ Bars:	3 per panel

Available Heights:	1.8m to 2.4m

Finish:	Any RAL colour. Marine coating finish available.

Panel Installation:	Max 50mm above ground level. Available with dig-in or base-plated post options.

Topping Options:	Cranked or straight extensions for mesh, barbed or razor wire

Posts: 	Supplied with plastic cap.

Post Dimensions:	100 x 50mm

Post Centres:	2,510mm

Post Testing:	All post heights tested to withstand wind speeds of up to 74mph (Force 12) in varying ground conditions.

Fixings:	41 x 21mm channel full length clamp bar, M8 x 140 cup square security bolt. M8 shear nut, M8 washer.

Warranty:	40-year guarantee on performance and appearance.

Notes:	Information correct at date of publication. Alexandra Security Ltd reserves the right to change specifications at any time. E&OE.